# Friction Modelling and Grey-Box Identification of Joint-1 of a 6-DOF Svaya Robotic Manipulator

## 1. Objective

The objective of this work is to identify a mathematical model capable of representing the dominant friction-related behavior observed in Joint-1 of a 6-DOF Svaya robotic manipulator using experimentally collected data.

The study aims to:

- Estimate physically interpretable friction parameters.

- Compare simple and advanced friction models.

- Investigate whether a data-driven neural network can improve prediction accuracy beyond classical friction models.

- Analyze the trade-off between physical interpretability and predictive performance.

## 2. Experimental Data

Experimental data were collected from the robot controller at a sampling frequency of 200 Hz (sampling period = 5 ms).

The available measurements are:

- Motor torque (Tm)

- Joint torque sensor output (Tj)

- Joint velocity (v)

- Joint position (q)

- Timestamp

Only Joint-1 data were considered in the present study.

The gearbox reduction ratio is:

N = 100

Joint position and velocity measurements were converted from degrees and degrees/second to radians and radians/second respectively.

## 3. Residual Torque Computation

The internal friction torque is not directly measurable.

Therefore a residual torque quantity was constructed using the available measurements:

Tf = N·Tm − Tj

where:

- Tm = measured motor torque

- Tj = measured joint torque

- N = gearbox ratio

This residual torque is treated as the target variable for modelling.

It is important to note that this quantity is not guaranteed to represent pure friction torque.

The residual torque may also contain contributions from:

- Inertial effects

- Gearbox dynamics

- Compliance

- Backlash

- Servo dynamics

- Controller dynamics

- Measurement noise

- Other unmodelled effects

Therefore the identified model should be interpreted as a model of the measured residual torque rather than a direct measurement of physical friction.

## 4. Data Partitioning Strategy

The dataset contains approximately 45,000 samples.

To avoid information leakage and preserve temporal structure, the data were not randomly shuffled.

Instead, a sequential split was used:

Training Set : First 60%

Validation Set : Next 20%

Test Set : Final 20%

This strategy evaluates the ability of the model to generalize to future unseen operating conditions.

## 5. Model 1 – Coulomb and Viscous Friction Model

The first model assumes that the residual torque can be represented using a combination of Coulomb and viscous friction.

The model is:

Tf(v) = Tc·tanh(v/Vs) + Bv·v

where:

Tc = Coulomb friction coefficient

Bv = viscous friction coefficient

Vs = smoothing velocity

The hyperbolic tangent function is used instead of the discontinuous sign() function to improve numerical optimization.

Model parameters are estimated by nonlinear least-squares fitting using the training data.

## 6. Model 2 – Stribeck Friction Model

The second model extends the Coulomb-viscous model by introducing the Stribeck effect.

The model is:

Tf(v) = [Tc + (Ts − Tc) exp(-( |v| / Vs )^α)] tanh(v/ε)

- Bv·v

where:

Ts = static friction coefficient

Tc = Coulomb friction coefficient

Bv = viscous coefficient

Vs = Stribeck velocity

α = shape factor

The Stribeck model captures the reduction in friction as velocity increases from zero and is widely used in robotic friction modelling.

Model parameters are estimated using nonlinear least-squares optimization.

## 7. Model 3 – Grey-Box Residual Learning Model

Although the Stribeck model captures major friction characteristics, some systematic errors may remain.

To model these effects, a grey-box architecture is introduced.

The total model is:

Tf = Tphys + Tnn

where:

Tphys = Stribeck friction model

Tnn = neural-network correction term

The neural network is trained only on the residual error:

Residual = Tf − Tphys

Input features:

- Joint position (q)

- Joint velocity (v)

- Motor torque (Tm)

- Joint torque (Tj)

Output:

- Residual correction torque

The neural network consists of:

Input Layer → 64 neurons

Hidden Layer → 64 neurons

Hidden Layer → 32 neurons

Output Layer → 1 neuron

Tanh activation functions are used in hidden layers.

The neural network is trained using the Adam optimizer and mean squared error loss.

## 8. Performance Metrics

Three metrics are used to evaluate model performance.

### Root Mean Square Error (RMSE)

Measures average prediction error magnitude.

Lower values indicate better performance.

### Mean Absolute Error (MAE)

Measures average absolute prediction error.

Less sensitive to outliers than RMSE.

### Coefficient of Determination (R²)

Measures the fraction of variance explained by the model.

R² = 1 indicates perfect prediction.

R² = 0 indicates no predictive capability.

## 9. Residual Analysis

Residual analysis is performed to evaluate model adequacy.

Residual = Actual − Predicted

The following plots are generated:

- Actual vs Predicted Torque

- Residual Error vs Sample Number

- Residual Histogram

- Velocity vs Friction Curve

A good model should produce:

- Small residual magnitude

- Zero-mean residuals

- No systematic patterns

- Approximately symmetric residual distribution

## 10. Model Comparison

The three models are compared using:

- Validation RMSE

- Test RMSE

- Validation MAE

- Test MAE

- Validation R²

- Test R²

The comparison allows evaluation of:

- Improvement obtained by introducing Stribeck friction.

- Improvement obtained by grey-box residual learning.

- Trade-off between physical interpretability and predictive accuracy.

## 11. Expected Outcomes

If the Stribeck model significantly improves performance over the Coulomb-viscous model, it indicates the presence of velocity-dependent nonlinear friction effects.

If the grey-box model further improves prediction accuracy, the residual signal likely contains additional dynamics not captured by conventional friction laws.

These may include:

- Gearbox effects

- Compliance

- Backlash

- Servo dynamics

- Controller dynamics

- Other unmodelled disturbances

The final comparison provides insight into how much of the measured residual torque can be explained using classical friction models and how much requires data-driven modelling.