# Nonlinear Least-Squares Parameter Identification for Friction Model Estimation

## 1. Introduction

A key objective of this study is to identify a mathematical relationship between the measured residual torque and joint velocity of a 6-DOF Svaya robotic manipulator. Since the true friction parameters of the robotic joint are unknown, a parameter estimation procedure is required to determine the values that best explain the experimentally observed behavior.

This parameter estimation problem is formulated as a nonlinear least-squares optimization problem and solved using the curve_fit() function available in the SciPy optimization library.

The identified parameters are subsequently used to construct friction models of increasing complexity, including Coulomb-Viscous and Stribeck friction models.

## 2. Parameter Identification Problem

The experimentally measured target variable is defined as

[ T_f = N T_m - T_j ]

where:

- (T_f) = measured residual torque

- (T_m) = motor torque

- (T_j) = joint torque sensor output

- (N) = gearbox reduction ratio

The residual torque is assumed to contain dominant friction-related effects together with other transmission and unmodelled dynamic contributions.

The objective of parameter identification is to determine the model parameters that minimize the discrepancy between the measured residual torque and the torque predicted by the friction model.

## 3. Coulomb-Viscous Friction Model

The first friction model considered in this work is the Coulomb-Viscous friction model:

[ T_f(v) = T_c ()

- B_v v ]

where:

- (T_c) = Coulomb friction coefficient

- (B_v) = viscous friction coefficient

- (V_s) = smoothing velocity parameter

- (v) = joint velocity

The hyperbolic tangent function provides a smooth approximation to the discontinuous sign function, thereby improving numerical stability during optimization.

The unknown parameter vector is:

[ = [T_c,;B_v,;V_s] ]

The objective is to determine the parameter values that best fit the experimental data.

## 4. Stribeck Friction Model

To capture nonlinear friction effects occurring near zero velocity, the Stribeck friction model is also investigated:

[ T_f(v) = ()

- B_v v ]

where:

- (T_s) = static friction coefficient

- (T_c) = Coulomb friction coefficient

- (B_v) = viscous friction coefficient

- (V_s) = Stribeck velocity

- () = shape factor

The unknown parameter vector becomes

[ = [T_s,;T_c,;B_v,;V_s,;] ]

The optimization procedure remains identical, but the parameter space becomes larger and more nonlinear.

## 5. Least-Squares Optimization Formulation

For every experimental sample (i), the friction model predicts a torque value

[ _{f,i} ]

which is compared against the measured torque

[ T_{f,i} ]

The prediction error is defined as

[ e_i = T_{f,i}

- _{f,i} ]

To estimate the model parameters, the sum of squared prediction errors is minimized:

[ J() = _{i=1}^{N} e_i^2 ]

or equivalently

[ J() = {i=1}^{N} ( T{f,i}

- _{f,i} )^2 ]

This objective function is known as the least-squares cost function.

The parameter estimation problem can therefore be written as

[ ^* = _{} J() ]

where (^*) represents the optimal parameter set.

The least-squares criterion is widely used in system identification because it penalizes large prediction errors more heavily than small errors and provides statistically consistent parameter estimates under Gaussian measurement noise assumptions.

## 6. Nonlinear Least-Squares Estimation

For linear models, analytical solutions can often be obtained directly. However, the friction models used in this work contain nonlinear functions such as

[ () ]

and

[ e^{-x} ]

which make the optimization problem nonlinear.

Consequently, iterative numerical optimization techniques are required.

The SciPy function curve_fit() solves the nonlinear least-squares problem by repeatedly updating the model parameters until the prediction error is minimized.

Starting from an initial parameter guess

[ _0 ]

the optimizer evaluates the model prediction, computes the associated least-squares cost, and iteratively modifies the parameters to reduce the objective function value.

The process continues until convergence is achieved.

## 7. Levenberg-Marquardt Optimization

The optimization procedure implemented by curve_fit() is based on the Levenberg-Marquardt algorithm for unconstrained nonlinear least-squares problems.

The algorithm combines characteristics of:

- Gradient Descent

- Gauss-Newton Optimization

The update rule can be expressed as

[ _{k+1} = _k

- (J^T J + I)^{-1} J^T e ]

where:

- (J) = Jacobian matrix

- (e) = error vector

- () = damping coefficient

The damping term allows the algorithm to transition smoothly between gradient-descent-like and Gauss-Newton-like behavior, improving convergence robustness.

## 8. Application to Friction Identification

For the Coulomb-Viscous model, the optimization estimates:

[ T_c,; B_v,; V_s ]

such that the predicted friction curve best matches the experimentally observed residual torque.

For the Stribeck model, the optimization estimates:

[ T_s,; T_c,; B_v,; V_s,;  ]

thereby capturing additional nonlinear friction effects near zero velocity.

The identified parameters are subsequently validated using independent validation and test datasets.

## 9. Interpretation of Identified Parameters

The estimated parameters should not be interpreted as direct measurements of physical gearbox friction coefficients.

The target quantity

[ T_f = NT_m - T_j ]

may contain contributions from:

- Friction

- Joint elasticity

- Gearbox compliance

- Servo dynamics

- Controller dynamics

- Measurement noise

- Transmission losses

Therefore, the identified parameters are more accurately interpreted as effective friction-model parameters that best describe the measured residual torque signal.

Despite this limitation, the identified models provide valuable insight into the dominant nonlinear torque characteristics present in the robotic joint and form the foundation for subsequent grey-box residual learning approaches.

## 10. Summary

The friction parameter estimation problem was formulated as a nonlinear least-squares optimization task. Unknown friction model parameters were identified by minimizing the squared difference between experimentally measured residual torque and model-predicted torque. The optimization was performed using SciPy’s curve_fit() implementation of the Levenberg-Marquardt algorithm. This approach enables physically interpretable parameter estimation while providing a baseline against which more advanced grey-box residual learning models can be evaluated.